Victor Reader

Stratus12 M

User Guide by HumanWare

Version 2-20110616

©Copyright 2011. All rights reserved, Technologies HumanWare Inc.

This manual is protected by copyright belonging to Technologies HumanWare Inc., with all rights reserved. According to copyright law, the manual may not be copied in whole or in part, without written consent from Technologies HumanWare Inc.

Every effort has been made to ensure that the information contained in this publication is correct at the time of issue. Neither Technologies HumanWare Inc. or its subsidiaries HumanWare USA Inc., HumanWare Europe Limited, or HumanWare Australia Pty Limited, nor its agents, assume any responsibility for errors or omissions. Nor is any liability assumed for damages resulting from the use of the information contained herein.

# Getting Started Session with Victor Reader Stratus12

Welcome to Victor Reader Stratus12, your new digital talking book player. The first time you use Victor Reader Stratus you should connect the player to a power outlet.

To connect to a power outlet, plug one end of the provided AC/DC power cord in the power jack. It is located near the right corner on the back side of the player. Plug the other end of the power cord in an electrical outlet. When first receiving the player or after replacing the battery or after an extended storage period, it is best to leave the player plugged into the power outlet overnight to ensure that the battery is fully charged.

The Power button may glow amber or green. The button may flash amber when the player is Off and plugged into a power outlet. The flashes indicate that the battery is recharging. When it stops flashing, the battery is fully charged. When the player is turned On, the Power button glows green, whether it is plugged in or not. Under regular circumstances, the battery takes up to 4 hours to recharge and will provide up to 10 hours of playtime for most DAISY books.

Please note that you can use Victor Reader Stratus while charging the battery.

The package contains the following items: A Victor Reader Stratus12, Optional Key pad cover (not on all units) that can be removed to expose the player’s extended navigation keys, a power cord, a plastic bag with documentation and this CD. This CD contains a short Getting Started session followed by the User's Guide.

You can pause and resume reading by simply pressing the Play/Stop key, which is located at the bottom center of the player between the Rewind and Fast-Forward keys.

We will now begin the Getting Started session. Place the player in front of you with the carrying handle on the left. Above the handle in the top left corner is a built-in speaker.

On the top center of the player are three pairs of up and down keys. From left to right, these are the Tone, Volume and Speed keys.

Beneath the Tone, Volume and Speed controls, you will find a telephone style numeric keypad. There is a raised dot on the number 5 key. You will use this numeric keypad to move through the structure of a book as well as enter bookmark or page numbers. Keys 2, 4, 6 and 8 will often be used as logical arrows used for navigation, representing Up, Left, Right and Down respectively. To the left of the 0 key is the Star or Cancel key and to the right of the 0 key is the Pound or Confirm key.

To the left of the twelve Numeric keys, about midway down the player, are three more keys aligned vertically. The top key is the Go to key that allows you to go directly to an item such as a page heading, file, or book number. Press it multiple times to choose which item. Immediately underneath the Go to key is the Bookmark key. The Bookmark key is used to go to, insert or remove bookmarks.  Press the Bookmark key multiple times to select the appropriate bookmark function. The third and lowest key is the Eject key. It is used to eject CDs from the CD drive slot.

To the right of the twelve Numeric keys is the Power key. It is recessed to prevent accidental pressing. The Power key is used to power the Stratus ON and OFF. Note that when the Stratus is off and you connect it to the power outlet, it will take a few moments before the Power key becomes operational.

Beneath the twelve Numeric keys is a group of 4 keys. Three of them are lined up along the lower edge of the player. These are, from left to right, the Rewind, Play/Stop and Fast Forward keys. Finally, above the Play/Stop key is the Sleep key. It is crescent shaped and has an orange LED indicator to its left. Press it once to hear the current time and date, or press it multiple times to set different time intervals after which the Stratus will automatically power off.

On the right side of the player, near the center, you will find from top to bottom the following components: an SD card slot, a USB port, and the earphones jack, which can also be used for an external speaker. Note that SD and USB components are not available on the Basic Stratus model.

Located on the back side of the player, slightly to the right of center, you will find the power input jack. It is used to connect the player to an electrical outlet.

On the front side of the player you will find the CD-ROM slot drive.

To insert and play a new book

Turn On Victor Reader Stratus by pressing and holding the Power key. You will hear a beep and a Welcome message.

Insert your CD or SD card. Gently push the CD forward. The drive mechanism will take over and finish inserting the CD. After a few seconds, Victor Reader Stratus will announce the title. Press the Play/Stop key to start playing the book.

NOTE: If you accidentally insert a CD upside down, it will be ejected within 15 seconds without damage.

You can adjust the Tone, Volume, and Speed by pressing the keys up or down. The Tone, Volume, and Speed control keys are located on the top center of the face.

To move back or forward at high speed in the book, press and hold the Rewind or Fast-Forward key until you reach the desired spot, then release. The normal play speed will be resumed.

To pause a reading session, simply press the Play/Stop key. To resume reading where you left off, press the Play/Stop key again. Note that if your player is not connected to a power outlet, and if it stays in pause mode for more than 30 minutes, it will shut Off automatically to save battery life.

To end a reading session, simply turn the player Off by pressing and holding the Power key until the player announces that it is closing. Victor Reader Stratus will remember where you left off in the book for your next reading session.

To start a new reading session in the same book, turn On your Victor Reader Stratus by pressing and holding the Power key. After a few seconds, you will hear the title of the book and a beep. Pressing the Play/Stop key will make the player resume where you left off in your last reading session.

To eject a CD

To eject a CD simply press the Eject key. Victor Reader Stratus will announce Eject and after a few seconds the motorized mechanism will gently slide the CD out of the drive.

Key Describer mode

When there is no book present, the Stratus is automatically set to Key Describer mode. In this mode, anytime a key is pressed, the player announces a short description of its function. This can be useful when learning how to use the player. If a book is present, you can still access the Key Describer by pressing and holding the Information key (0). Press and hold the Information key (0) again to exit Key Describer mode.

This completes the Getting Started session. The User's Guide will begin playing momentarily.

User’s Guide

# About Victor Reader Stratus12

HumanWare is proud to introduce Victor Reader Stratus12, the simple and friendly digital talking book player.

Victor Reader Stratus12 was designed to suit all your reading needs, from leisure reading such as best-sellers, novels, and magazines to more complex structured works such as reference books. The Stratus allows you to easily navigate within the structure and quickly go to desired information.

With its simple to use functions, you can consult the table of contents, go to a section, or return to a bookmark.

We hope that you will enjoy countless hours of reading using your new digital talking book player!

About HumanWare

HumanWare (http://www.humanware.com) is the global leader in assistive technologies for the print disabled. HumanWare creates products for people who are blind, have low vision and/or have learning disabilities. The innovative range of HumanWare products includes BrailleNote, the leading productivity device for the blind in education and business, as well as for personal use; the Victor Reader line, the world's leading digital audiobook players; myReader2, HumanWare’s unique "auto-reader" for people with low vision, SmartView Synergy SI desktop magnifier and the Trekker Breeze GPS orientation aid.

Important Safety Instructions

Follow all warnings, cautions, and instructions marked on the product.

Carefully read the instructions described in this user guide prior to using the Stratus.

Only use or store the Stratus on stable and solid surfaces.

Keep away from water or high humidity. Do not submerge.

Only use cables provided with the Stratus.

The power supply cord is a disconnect device for pluggable equipment. The socket-outlet should be installed near the equipment, and be easily accessible.

This product should be operated from the type of power indicated on the marking label. If you are not sure of the type of power available, consult your dealer or local power company.

Refer all servicing of this product to qualified service personnel. Do not attempt to service it yourself, as opening or removing covers may expose you to dangerous voltage points or other risks.

The above exclamation point within an equilateral triangle is intended to inform the user to the presence of important operating and maintenance (servicing) instructions in the literature accompanying the appliance.

Battery Safety precautions

- Do not use or leave the battery near heat source and fire. Do not store in high temperatures

- Only use the Stratus power adapter to charge the battery

- Do not disassemble or modify the battery

- Do not short circuit the terminals

- Do not immerse the battery in water, do not allow it to get wet

- Do not strike or throw the battery

- Do not pierce or strike the battery with sharp objects or a hammer

- Do not solder the battery

- Do not reverse the positive and negative terminals

- Do not use the battery for purposes other than those intended by manufacturer

- Do not use the battery with primary battery or batteries of different capacities or brands

- If the battery leaks and liquid gets into the eyes, do not rub eyes. Instead, rinse the eyes with clean running water and seek immediate medical attention to prevent injury

- If the battery leaks and comes into contact with skin, immediately rinse area with clean running water to prevent injury

Medical Compliance Warnings (if applicable)

The following text only applies if you purchased the Stratus as a medical device. This product is equipped with a 3-wire grounding type plug as a safety feature. This plug has a third (grounding) pin and can only be plugged into a grounding-type power outlet. If necessary, contact an electrician to replace your power outlet with a grounding-type power outlet.

This product should be operated with the external power supply indicated on the marking label to ensure compliance with the requirements of IEC 60601-1. The external power supply must be of the following manufacturer and model:

Manufacturer:		GlobTek, Inc.

Model Number:		GTM9100P10012

Electromagnetic compatibility: The use of accessories and cables other than those specified by HumanWare may result in increased emissions or decreased immunity of the Stratus.

Placement of equipment: The Stratus should not be used adjacent to or stacked with other equipment.

Headphone connector shall be connected to medical grade SELV circuits only or they shall be connected to medical grade isolating devices.

FCC Warning

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

—Reorient or relocate the receiving antenna.

—Increase the separation between the equipment and receiver.

—Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

—Consult the dealer or an experienced radio/TV technician for help.

Notice for Canada

This Class B digital apparatus meets all requirements of the Canadian Interference-Causing Equipment regulations.

Disposal

At the end of the life of the Victor Reader Stratus, its internal components must be disposed in compliance with the local authorities.

The Victor Reader Stratus contains no hazardous materials. For disposal, return to HumanWare or follow local governing ordinances or hospital procedure.

Table of Contents

Getting Started Session with Victor Reader Stratus12	i

About Victor Reader Stratus12	iv

1.	Overview of Victor Reader Stratus12 M	1

1.1	Unpacking the Player	1

1.2	Stratus12 M options	1

1.3	Optional keypad cover	1

1.4	Physical Description of Victor Reader Stratus	3

1.4.1	The Face of the Player	3

1.4.2	The Left Side of the Player	4

1.4.3	The Right Side of the Player	4

1.4.4	The Front Side of the Player	4

1.4.5	The Back Side of the Player	4

1.5	Charging the Battery	4

1.6	Powering On/Off	5

1.7	Inserting or ejecting a CD	5

1.8	Inserting or Removing the SD Card	5

1.8.1	SD Card Voice Label	6

1.9	Book Detection	6

1.10	Bookshelf Structure	6

1.11	Other Reserved File Names	7

2.	Basic Functions	8

2.1	Audio Controls	8

2.2	Play/Stop	8

2.3	Rewind and Fast Forward	8

2.4	Confirm and Cancel — Pound and Star keys	8

2.5	Go To	8

2.6	Key Describer Mode	8

3.	Numeric Key Functions	9

3.1	Description of Keys	9

3.2	Navigation Keys	10

3.2.1	Undo navigation	10

3.3	Bookshelf	10

3.4	Time Jump Navigation Mode	10

3.5	Ten page Jump	11

3.6	Text-to-Speech Navigation Elements	11

3.7	Spell Mode for Text Bookshelf	11

3.8	Skippable elements	11

3.8.1	Recall skippable elements	11

3.9	User Guide — Bookshelf Key (Press and hold)	12

3.10	Bookshelf Navigation — Key 1	12

3.10.1	Multi-Level Bookshelf Navigation	12

3.11	Deleting Books- Key 3	12

3.12	Copying books from CD or USB flash drives – Key 3	12

3.13	Where am I ? — Key 5	12

3.14	Multi-Voice Text to Speech (TTS) – Key 7	13

3.15	Playback mode (DAISY books) –Key 9	13

3.16	Random Music Playback Mode — Key 9	13

3.17	Information — Key 0	13

3.17.1	Available Information Items	13

4.	ADVANCED FUNCTIONS	14

4.1	Sleep Mode and Time announcement	14

4.2	Setting the Date and Time	14

4.3	Bookmarks	15

4.3.1	Insert Bookmark	15

4.3.2	Highlight Bookmark	15

4.3.3	Bookmark List	16

4.3.4	Remove Bookmark	16

4.3.5	Bookmark alert	16

4.4	Text Search	16

4.4.1	Search Next or Previous	17

4.4.2	Search for Current Word	17

5.	Bookshelf Structure and Characteristics	19

5.1	Talking Books	19

5.1.1	Talking Books Structure	19

5.1.2	Talking Books Characteristics	19

5.2	Other Books	20

5.2.1	Other Books Structure	20

5.2.2	Other Books Characteristics	20

5.3	Music Bookshelf	21

5.3.1	Music Structure	21

5.3.2	Music Characteristics	21

5.3.3	Playlists	22

5.3.4	Temporary Playlist	22

5.3.5	Folder and File Name Announcement	22

5.4	Podcasts Bookshelf	22

5.4.1	Podcast Structure	23

5.4.2	Podcasts Characteristics	23

5.5	Text Bookshelf	23

5.5.1	Text File Structure	23

5.5.2	Text File Characteristics	23

5.5.3	Navigating HTML/XML/DOCX Headings	24

6.	Configuration Menu – Key 7	25

6.1	Menus and Menu Items List	25

6.1.1	Playback	26

6.1.2	Skip	26

7.	Battery Replacement Procedure	27

7.1	Removing spent battery	27

7.2	Inserting new battery	27

8.	Updating Stratus Software	28

9.	Troubleshooting	29

9.1	Book Navigation	29

9.2	Battery and Charging	29

9.3	CDs	29

9.4	General	30

10.	Technical Specifications	32

11.	Customer support	33

12.	Manufacturer Warranty	34

13.	End User License Agreement	35

14.	Appendix 1 - Status Error Messages	36

# Overview of Victor Reader Stratus12 M

## Unpacking the Player

The package contains the following items: A Victor Reader Stratus12 M, Optional Keypad cover (not on all units), which may be removed to expose the player’s navigation keys, a power cord, a plastic bag with documentation and a CD. This CD contains a short Getting Started session followed by the User's Guide. You can print the User’s Guide & Getting Started session if required.

Before using Stratus for the first time, you should connect the player to a power outlet to charge the battery. You can use your Victor Reader Stratus while charging the battery.

If any of the above items are missing in the package please contact HumanWare:

Toll Free: 1 888 723-7273 (Canada & U.S.A.)

Telephone: 1 (819) 471-4818

Fax: 1 (819) 471-4828

Website: www.humanware.com

North America:  the package contains a card to fill out if you wish to purchase an optional extension to your basic warranty. If you find this card and are interested in the extended warranty please note that the card must be returned within 30 days. If you have difficulty filling out the card or wish more information please contact HumanWare:

E-mail: us.info@humanware.com

## Stratus12 M options

The Stratus M (multimedia) player can play content found on CDs, SD cards and USB flash memory. Stratus M may also have an optional accessory to access books on the Internet through either a Wi-Fi or 3G connection. On some Stratus M models the CD drive is not present. Therefore, if your Stratus M has no CD, disregard any mentions of CDs or the CD Drive. To confirm which model you have, press key 0 (Information.) Unless stated otherwise, this user guide will refer to all Stratus12 M models as simply the “Stratus”.

## Optional keypad cover

Your player may have an optional key cover installed. The cover prevents use of book navigation keys for those users who may not wish to use them. If you wish to take full advantage of the book navigation features of your Victor Reader Stratus then you may remove the cover.

The keypad cover is attached at the four corners of the player. The cover is held in place by a tab on each corner.

The Keypad cover prevents the use of the following keys: Go to, bookmark, and the numeric keys.

If you wish to remove the keypad cover:

-Hold the player by the carrying handle using your left hand.

-Using your right hand, locate the top right corner of the cover. The tab is in the middle of the corner.

-Place the side of your right index finger underneath the tab.

-Using your thumb as leverage on the side of the player, apply pressure to slightly lift the tab from the side of the player.

-Using same method, slightly lift the tab on the lower right corner of the cover.

-With both corners raised, you should be able to lift the cover off. If needed, lift the bottom left corner and remove the keypad cover.

If you wish to reinstall the keypad cover, you will have to align the cover properly over the Stratus, making sure the carrying handle of the Stratus and that of the cover are overlapping.

Then press down on the cover until all four corners snap into place.

## Physical Description of Victor Reader Stratus

### The Face of the Player

Figure 1: Stratus keys

Stratus buttons and features:

- Tone controls

- Volume controls

- Speed controls

- Numeric keys

- Go to

- Bookmark

- Eject

- Power

- Rewind

- Play/Stop

- Fast Forward

- Sleep

- SD card slot

- USB port

- Earphone jack

- Power input jack

- CD Drive slot

The built-in speaker is located on the upper left corner of the face.

On the top center, there are three pairs of up and down keys. From left to right, these are the Tone, Volume and Speed controls.

Beneath the Tone, Volume and Speed controls, you will find the twelve Numeric keys allowing you to easily move through the structure of a book. The numbers and Star and Pound keys are in the same position as a standard telephone keypad.

To the left of the twelve Numeric keys, about midway down the player, are three more keys aligned vertically. The top key is the Go to key, which allows you to go directly to a desired page.  Immediately underneath the Go to key is the Bookmark key. The Bookmark key is used to highlight sections or to mark important positions in a book for future reference. Press the Bookmark key multiple times to choose to insert, go to, or remove bookmarks. The third and lowest key is the Eject key. It is used to eject CDs from the CD drive slot. The Eject key is also used to return books borrowed from a Daisy online service (for players with internet access only).

To the right of the twelve Numeric keys is the Power key. It is slightly recessed to prevent accidental pressing. The Power key is used to power the Stratus ON and OFF.

Beneath the twelve Numeric keys is a group of 4 keys. Three of them are lined up along the lower edge of the player. These are, from left to right, the Rewind, Play/Stop and Fast Forward keys. Finally, above the Play/Stop key is the Sleep key. It is crescent shaped and has an orange LED indicator to its left. The Sleep key is used to announce the time and activate the sleep timer. You press this Sleep key multiple times to announce the time or to set different Sleep time periods after which the player will automatically shut off.

### The Left Side of the Player

On the left side of the player you will find the built-in handle and speaker.

### The Right Side of the Player

On the right side of the player, near the center, you will find from top to bottom the following components: an SD card slot, a USB port, and the earphones jack, which can also be used for an external speaker.

### The Front Side of the Player

On the front side of the player you will find the CD-ROM drive slot.

### The Back Side of the Player

Located slightly to the right of center on the back of the player you will find the power input jack. It is used to connect the player to an electrical outlet in order to recharge the battery.

## Charging the Battery

The battery is rated for approximately 300 recharge cycles. Under normal conditions the fully charged battery can last up to 10 hours for continuous play of DAISY books. However, the battery playtime may be reduced in the following cases:

If the battery is charged for less than 4 hours.

Extensive use of book navigation commands.

Non-DAISY CDs take more power and play for about 3 hours on battery power.

High Volume Level or High Speed reading.

Connection to the internet.

The battery will automatically recharge whenever the player is plugged to AC power. The player can be used while recharging. To recharge the battery simply connect it to AC power. If the player is on when you connect, you will hear a short chime confirming that power is connected. You will hear the chime again when disconnecting from the AC power outlet. However, the player will recharge regardless of whether it is on or off.

For instructions on how to replace the battery, please consult the Battery Replacement Procedure section of this user guide.

NOTE: When the player is not connected to a power outlet, and staying in pause mode for more than 30 minutes, it will shut Off automatically to save the battery.

## Powering On/Off

To turn the player On, press and hold the Power key. You will hear a beep and a welcome message.

To turn the player Off, press and hold the Power key again. You will hear two beeps indicating the player is powering Off. Note that when the Stratus is off and you connect it to the power outlet, it will take a few moments before the Power key becomes operational.

The Power button has the following behavior:

- The player is active. The button is green regardless whether the battery is charging or not charging.

- The player is off, plugged to AC power and recharging. The Power button is amber and blinking.

- The player is off, plugged to AC power and not recharging. The Power button is amber and not blinking.

- The player is off, not plugged to AC power. The Power button does not glow.

## Inserting or ejecting a CD

Insert your CD in the slot located on the front side of the player. Then gently push the CD forward. A motorized mechanism will slide the CD in the drive. After a few seconds, Stratus will announce the title of the book. Pressing the Play/Stop key will start reading the book. If you accidentally insert a CD upside down, it will be ejected after a brief time without damage.

To eject a CD, simply press the Eject key, located in the bottom left corner. A motorized mechanism will gently eject the CD from the drive.

NOTE: The Eject key can only work when the Stratus is powered On.

## Inserting or Removing the SD Card

To insert and remove the SD card follow these steps:

Have the player in front of you on a stable surface. The handle should be on the lower left side. The SD card slot is near the center of the right edge.

Hold the SD card with your right hand.

-One corner of the SD card is cut at a 45 degree angle. This cut corner should on the upper left corner. The card's connectors, the numerous small raised ridges, should be along the left side, underneath the card. This is the side that you will insert into the player's SD card slot. If the card is not oriented properly it will resist insertion.

Gently push the card down into the slot until it clicks into place. Do not use excessive force. Only slight pressure is required to lock the SD card into place.

-To remove the card, push down on the top edge until it clicks again to release it. Then pull it out.

Note that SD cards have a write-protection tab that can prevent accidental deletion of its contents.  When the write protect tab is pushed towards the connectors, the SD card is not write protected, meaning that content can be added or deleted from the SD card. When the tab is pushed away from the connectors, the SD card is write-protected, meaning that content cannot be added to or deleted from the card.

### SD Card Voice Label

Attaching print or Braille labels to SD cards is not possible as it can damage the Stratus. To help you identify SD cards, the Stratus supports an optional voice label. The label is stored in a reserved text file named $VRLabel.txt saved on the root of the SD card. You may use Windows Notepad to create the file. Whenever you insert the card or turn on the Stratus, it will look for this label file. If found, Stratus will announce its contents. Note that the label cannot exceed 50 characters. Stratus will ignore any characters after the first 50 in the label file. This label file is optional. If you do not require an auto-announcing label for your SD card there is no need to set it up. This label has nothing to do with the label that you see when viewing the SD card Properties in Windows.

## Book Detection

When the Stratus is powered on, it will look for the last book read. If this book is not found, it will open the first book it finds in the available media according to the following order: SD card, USB, Internal memory and CD.

## Bookshelf Structure

The Stratus recognizes various book types which are saved in separate folder structures called Bookshelves. You select the Bookshelf you want to read with multiple presses of the Bookshelf (1) key. You move backward and forward along each Bookshelf using the 4 and 6 keys. The Bookshelf list is circular. On the SD or USB source each Bookshelf is stored within a reserved folder name beginning with “$VR”. Within each of these reserved folders (Bookshelves) there may be user defined subfolders containing separate books or categories of files. You may put other folders and files on the SD card but it is only the contents of the $VR reserved folders that define the Stratus bookshelves. Note that when you insert a card that is not write-protected, Stratus will automatically create the $VR reserved folders.

The Bookshelf structure and usage are defined as follows:

Bookshelf: Talking Books ($VRDTB folder).

Book types: DAISY 2.x, NISO Z39.86, NIMAS 1.1 EPUB

Usage: The files comprising each of your DAISY or NISO books should be saved in separate subfolders under this folder. Use the 2 / 8 keys to select folder or book navigation level and keys  4 / 6 to navigate at the chosen level. Press PLAY to select the book. In this bookshelf the current reading position and bookmarks are saved separately for each book.

Bookshelf: Other Books ($VROtherBooks folder).

Book types: Non-DAISY or non-NISO recorded books composed of any of the Stratus supported audio file types.

Usage: The files comprising each book should be saved in separate subfolders under this folder. Folders may be nested. Optionally, single-file books may be directly placed in the root folder. Use the  2 / 8 keys to select folder or book navigation level and keys  4 / 6 to navigate at the chosen level. Press PLAY to select the book. In this bookshelf the current reading position and bookmarks are saved separately for each book. Playback stops at the end of the last file of each book.

Bookshelf: Music ($VRMusic folder).

Book types: Playlists (M3U type only) and recorded files composed of any of the Stratus supported audio file types.

Usage: A music book is either a playlist or the entire music structure (referred to as All Music book). The current reading position and bookmarks are saved separately for each book. You navigate between music books using the 4 and 6 keys on the bookshelf. The All Music book consists of nested folders containing audio files. For example, the structure could be genre, artist, album, and tracks. Navigate the All Music book using 2 / 8 keys to choose folder or file navigation level and then move within that level using 4 / 6 keys. Playback continues from the last file of one folder to the first file of the next folder. Random play mode (set by pressing 9) is available for Music books.

Bookshelf: Podcasts ($VRPodcasts)

Book types: Any of the Stratus supported audio file types.

Usage: Each podcast file is defined as a separate book meaning you must move from file to file using the 4 / 6 keys while in the Bookshelf. Files may be in subfolders in which case you use keys 2/8 to select folder or book navigation level. Reading position and bookmarks are saved separately for each book (file). Playback stops at the end of each book (file).

Bookshelf: Other audio

Book Type: Audio (CDA files) or Data (Any of the Stratus supported audio file types.)

Usage: Commercial audio or MP3 CD. You can navigate from file to file using the 4 and 6 keys. Random play mode (set by pressing 9) is available for this bookshelf.

Navigate by using keys 2 / 8 to choose folder or file navigation level and then move within that level using 4 / 6 keys.

Bookshelf: Text ($VRText folder).

Book types: Text files (bra, brf, docx, html, rtf, ses3, txt, xml)

Usage: Each text file is defined as a separate book meaning you must move from file to file using the 4 / 6 keys while in the Bookshelf. Files may be in subfolders in which case you use keys 2/8 to select folder or book navigation level. Reading position and bookmarks are saved separately for each book (file). Playback stops at the end of each book (file). Note: If your braille file (.brf) is not playing correctly, check that you have selected the correct braille translation table in the configuration menu (Key 7).

Note: Folder names are shown in mixed case for readability but Stratus does not require mixed case.

## Other Reserved File Names

The Stratus may create other file names beginning with “$VR” on the SD card. Removing or altering these files could result in unpredictable behavior.

# Basic Functions

## Audio Controls

Each of the Tone, Volume, and Speed controls has an Up and Down key. There is a beep to mark the upper and lower range of each control. If the book is not playing (or the key cover is attached) Stratus will announce the setting position. For tone and speed there is also a beep to mark the normal or 0 position. This indicates either a flat tone setting or normal speed setting.

## Play/Stop

To start playing a book, press the Play/Stop key.

To stop playing a book, press the Play/Stop key again.

## Rewind and Fast Forward

These keys allow you to go backward or forward quickly. You will hear the recording at a faster speed.

Press and hold the Rewind or Fast Forward key until you reach the desired spot, the movement of the Rewind or Fast Forward keys will accelerate the longer you hold the key down. For each few seconds you hold the keys you will move forward or backward 1 minute, 2 minutes, 5 minutes, 10 minutes, 15 minutes and so on. The largest jump is 5 minutes. Stratus will announce the time jumps and play a normal speed clip between each time jump. You can also press the Fast Forward and Rewind keys once to jump 5 seconds forward or backward or to jump to the next or previous line in text-based documents.

## Confirm and Cancel — Pound and Star keys

The Pound key allows you to confirm an operation or end a numeric entry such as a page number or a text entry such as a text search item.

The Star key allows you to cancel an operation.

The Pound and Star keys are located on the bottom row of the numeric keypad, to the right and left of the 0 key respectively.

## Go To

The Go To key is used to jump directly to a specific item such as a page, heading, folder, file, percentage, or book.

Press the Go To key multiple times to select which item you wish to go to, then enter the desired number. Press Pound (Confirm) to jump to that location, or press Play to jump and then begin reading from that location.

Note: when prompted for a number, press 0 or Rewind to jump to the beginning of the book, or press Fast Forward to jump to the end of the book.

## Key Describer Mode

With the Key Describer feature, the user can learn the function associated with the Stratus keys.  This is especially helpful when learning how to use the player.

If no book is present: The Key Describer mode will be activated automatically. If a book is present, you can activate the Key Describer mode by pressing and holding the Information (0) key. Press and hold the Information (0) key   again to exit Key Describer mode.

While in Key Describer mode you may press any key to hear its function.

# Numeric Key Functions

## Description of Keys

2

Navigation

Element

1

Bookshelf

3

Copy/Delete

4

Back

5

Where am I?

6

Forward

9

Playback mode

7

Menu

8

Navigation

Element

#

Confirm

0

Info

*

Cancel

- 1: Bookshelf

- 2: Up/ Navigation element

- 3: Copy/Delete book

- 4: Left/Back

- 5: Where am I?

- 6: Right/Forward

- 7: Menu

- 8: Down/ Navigation Element

- 9: Playback mode (DAISY books) or Random mode toggle (Music)

- Star key: Cancel

- 0: Information if pressed, Key Describer Mode if pressed and held

- Pound key: Confirm

## Navigation Keys

8

Down/Nav. Element

6

Right

2

Up/Navigation

Element

4

Left

The 2, 4, 6 and 8 keys have embossed edges pointing in the direction of the function they represent:  Up, Left, Right and Down respectively. These keys are navigation keys that allow you to easily go from one structural element (also called navigation level) to another to quickly find desired information.

Use the 2 and 8 keys to choose a navigation level. These differ from book to book but typically level 1 means chapter, level 2 means section, and level 3 means sub-section and so on. The page element may not be present on all books. The phrase level is usually the smallest navigation element but this is defined by the book producer. Stratus limits phrase jumps to a maximum length of 1 minute. First, choose a navigation level (chapter, page, etc.), using the 2 or 8 keys. Then use the 4 and 6 keys to move backward and forward between elements at the selected level. The 2 and 8 keys will announce only the levels available for the book. The phrase level is always provided but its extent depends on the book producer.

### Undo navigation

Pressing Star (Cancel) within 10 seconds of any single move backwards or forwards using keys 4 or 6 or single go to page move will undo the operation and return you to your previous position.  The Undo function does not apply to the Rewind or Fast Forward buttons.

## Bookshelf

If there are multiple titles available on your media then you may press key 1 to access the book list. The Left and Right arrows (keys 4 and 6) allow you to move from title to title to choose the book you want. Press the Go To key to enter a relative book number. End the book number entry with Pound (Confirm key) to move to the entered book number and remain on the bookshelf. When you've reached the desired book press Pound (Confirm key) or Play to open it. Pressing Play will have Stratus start to play from where you left off in the selected book.

## Time Jump Navigation Mode

Time Jump navigation can be selected using the 2 and 8 keys.

Press the key 4 to move Backward or the key 6 to move Forward by the jump interval.

The Time Jump interval can be set at 1, 5, 10 or 30 minutes in the Navigation and Playback section of the configuration Menu (key 7).

## Ten page Jump

The 4 (Left) and 6 (Right) keys allow you to move in 10-page jumps to quickly reach a specific page.

Select Page Navigation element by pressing the 2 (Up) or 8 (Down) key until you hear "Page".

Then, press and hold the 4 (Left) or 6 (Right) key. You will hear Victor Reader Stratus skip backward or forward in 10-page jumps.

Release the key when you are close to the desired page, then forward a page at a time by pressing the 4 (Left) or 6 (Right) keys successively until you reach the desired page. If your book does not offer page navigation then the "Page" element will not be announced when you press the 2 (Up) or 8 (Down) keys.

## Text-to-Speech Navigation Elements

When Stratus is in Text-to-speech mode there will be additional text navigation levels added to the 2 / 8 (up / down) key rotation. These include the text elements: screen, paragraph, line, sentence, word, spell and character. Note that if you do a search in a text then a Search level is added to allow search of the next or previous search term. Screen and line elements are defined arbitrarily as 25 lines per screen and a maximum of 80 characters per line. As with other navigation levels, you move between elements of the selected level using the 4 / 6 (left / right) keys. Note that the Screen element will only be available when no pages are defined in the book.

## Spell Mode for Text Bookshelf

Spell mode can be selected using the 2 and 8 keys. The Spell mode can be found between the Word and character navigation levels. Use keys 4 and 6 to move word by word. Stratus will announce the word and spell it. Note that capital letters are indicated and that words will be spelled at normal speed, regardless of TTS speed setting.

## Skippable elements

Stratus supports the following skippable elements: Footnote, Page number, Production note and Sidebar.

Skippable element can be set as Play, Skip or On Demand, see the Configuration Menu chapter for details. When a skippable element is encountered during playback, Stratus behaves as follows:

Play: The element is always played.

Skip: The element is never played.

On Demand: The element is not played but added to the recall list and the element’s name is announced. Note that the On Demand option is not available for page and line number elements.

### Recall skippable elements

On Demand skippable elements can be recalled with the Pound key.

Stratus remembers up to 5 skipped On Demand elements. The last added element will be the first to be recalled. Pressing the Pound key multiple times will recall the prior element in the list. When an element is recalled, the name of the element is played and playback starts at the beginning of the element. When the end of the element is reached, the message “Return to text” is played and the book position will return to where it was when the Pound key was pressed.

During the playback of a recalled element, the user may press Star (Cancel key) to quit playing the recalled element and return to the main text of the book.

## User Guide — Bookshelf Key (Press and hold)

You can access the built-in user guide at any time by pressing and holding key 1. Press and hold 1 again to exit.

## Bookshelf Navigation — Key 1

Press the Bookshelf (1) key multiple times to rotate through the circular list of Bookshelves. Empty bookshelves will not be announced. Bookshelves contain different types of books and files as defined in the section, Bookshelf Structure. You move backward and forward along a Bookshelf using the 4 and 6 keys. Press the Go To key to enter a relative book number. End the book number entry with the Pound (Confirm key) to move to the entered book number and remain on the bookshelf. When you reach the desired book press Pound (Confirm key) or Play to open it. Press Play to have Stratus start to play from where you left off in the selected book.

### Multi-Level Bookshelf Navigation

Optionally, you can create a multi-level folder structure for the Talking books, Other Books, Podcasts, and Text File bookshelves. Instead of only cycling through books one at a time with keys 4 and 6, you can choose the level of navigation with the UP and DOWN keys 2 and 8. Then, when pressing keys 4 or 6, you will move from folder to folder at the chosen level in a circular fashion. When you arrive at the folder of interest, press keys 2 or 8 again to select the book level within that folder and proceed to select the desired book. Navigating at the book level will also move from folder to folder. The Stratus will navigate up to 8 levels of folders under the $VR folder. If you have more than 8 levels, the additional levels will be recognized by the Stratus as level 8. If you also put files in the root of the $VR folder, those files will only be listed at the Book level of the 2/8 rotation.

Note: Multi-level bookshelves are not mandatory. If you choose not to organize books at multiple subfolder levels, you can just browse the bookshelf with keys 4 and 6 to navigate at the Book level.

## Deleting Books- Key 3

When navigating the bookshelf, you can delete a book on the SD card (except the All music book) by pressing key 3.

## Copying books from CD or USB flash drives – Key 3

You may copy DAISY and NISO books from a CD or external USB flash drives to the SD card. To do so, start playing the CD or USB book, then press key 3 to start the copying process. The percentage progress of the copy will be announced. It will take about one minute to copy 50mb, or about 3 minutes for the average book. The copying process can be cancelled at any time by pressing the Star key. If you copy a multi-CD book, the Stratus will create separate books on the SD card, one for each CD.

## Where am I ? — Key 5

The Where am I key will announce your reading position without interrupting the reading process. Depending on the book type, Stratus will announce one or more of the page and heading numbers, section title, or file name. For Music and Other audio, it will announce current folder and file names and the file times. For Talking books, Other Books, Audible, Podcasts it will announce the percentage of the book played as well as individual file times. For text files, it will announce the percentage of the file played. Reading will resume once this information is given.

## Multi-Voice Text to Speech (TTS) – Key 7

Press and hold key 7 to select a different voice from the available choices.

## Playback mode (DAISY books) –Key 9

For DAISY books that include both text and recorded audio, you may switch back and forth between the recorded and text portions of the book by pressing key 9. This could be useful if you wish to learn the spelling of a word or search for a specific phrase.

## Random Music Playback Mode — Key 9

When playing Music you can press key 9 to enable random play and press key 9 again to disable random play. If you are playing a permanent or temporary playlist only that playlist will be randomized. Note that the Random feature is only available if your music is stored in a folder called $VRMusic.

## Information — Key 0

The Information (0) key announces book, player, and battery information. There are two ways to access the information.

Press the Information (0) key. All information items will be announced. Or press the Information (0) key and press key 4 or 6 to move backward or forward through the items. You can also press the Information (0) key repeatedly to move through each item.

To cancel the Information messages, press the Star or Play keys.

You can also press and hold the Information key to enter and exit Key Describer Mode.

### Available Information Items

Book information items vary according to book type. Typically, information will include the following:

Book title

Number of files and folders (Music Bookshelf only)

Number of pages and total time

Total Headings in the book

Playtime elapsed and playtime remaining

Number of Bookmarks set in the book

Number of Books

Whether the player is operating on battery or AC power, if on AC power, whether the battery is charging

Current Battery Percentage

VICTOR READER Stratus software version number, Stratus model name, number of user authorization keys (if any), and serial number.

Note that the serial number is also written inside the battery compartment.

# ADVANCED FUNCTIONS

## Sleep Mode and Time announcement

The Sleep button allows you to obtain time announcements and set the Sleep Timer.

Press once to hear the time and date.

The Sleep feature allows the Stratus to be set to automatically power off after different lengths of time: 15minutes, 30 minutes, 45 minutes, 60 minutes, custom. The Custom option lets you enter the desired number of minutes using the number pad. End your entry with Pound (Confirm key) or Play.

Press Sleep multiple times to select your desired Sleep time. To deactivate the Sleep function, press Sleep repeatedly until you reach the OFF option.

When the sleep timer is active, an orange light located to the left of the Sleep button is turned ON. One minute before the timer powers off the player, the light will start blinking and you will hear a message informing you that 1 minute of Sleep remains. When the timer expires, the player announces that it will power off shortly, then does so without beeping.

## Setting the Date and Time

To set the date and time, press and hold the Sleep key. Use the 2 (Up) and 8 (Down) keys to navigate the menu. Date and time menu items are as follows: Set Time, Set Date and Set Advanced Time Settings. When on the menu item of your choice, press 6, Pound (Confirm key) or Play to enter that given menu. This menu will have various submenu items. Enter the correct value for each submenu using the numeric keys. Press Pound (Confirm key) or Play to move to the next submenu item or press Star (Cancel) to return to the menu. Note that if an invalid entry is made, you will be asked to enter a new value. When you reach the last submenu, press Pound (Confirm key)  or Play to confirm your settings and return to the date and time configuration menu.

You can exit the menu system at any point by pressing the Sleep key.

List of menus and submenus

Set Time: Hour, AM/PM (if on 12 h format), Minutes

Set Date: Year, Month, Day

Set Advanced Time Settings: 12h/24h format, Daylight saving setting

Note that the advanced time settings are modified by using the up/down navigation arrows (keys 2 and 8).

The daylight saving setting is a quick way to manually adjust your clock forward or back one hour. You can manually select Standard Time or Daylight Saving Time in the Advanced Time Setting submenu. Note: This setting does not make the player automatically change the time at the predetermined date twice a year.

Example:

If you wish to modify the current month:

Press and hold Sleep key until you hear the menu prompt.

Press 8 once to reach the Set Date menu.

Press Pound (Confirm key) twice to reach the Month submenu.

Enter the month (i.e. 1 to 12) using numeric keys.

Press Pound (Confirm) to move to the next submenu (day).

If you do not wish to modify the day, press Pound (Confirm) again to reach the end of the Set Date menu, at which point any settings entered in the Set Date menu will be saved.

Press the Sleep button once to exit the Time and Date settings menu.

## Bookmarks

The Bookmark key marks positions in the book so you can return to them later.

NOTE: Bookmarks and current position are always saved separately for each book.

Press the Bookmark key once or until you hear ‘go to bookmark’. Enter the number of the bookmark you wish to go to. Press the Pound key. Stratus will go to the bookmark and announce its number. Or, press the Play/Stop key. Stratus will go to the requested bookmark and begin playback.

### Insert Bookmark

Press the Bookmark key twice or until you hear ‘insert bookmark’. You can also access this function by holding the Bookmark key down instead of pressing it twice. Enter the number from 1 to 99998 that you wish to assign to the Bookmark. Press the Pound key to confirm or the Play/Stop key to confirm and start playback. The bookmark will be stored in memory.

NOTE: If you confirm without entering a number, Stratus will assign the first available bookmark number.

### Highlight Bookmark

Highlight Bookmarks are used to define the Start and the End positions of a passage for later playback. Setting highlight bookmarks is a great way to study important passages in text books.

#### Start Highlight Bookmark

Press the Bookmark key, 3 times or until you hear “Start highlight bookmark”.

Enter a bookmark number followed by the Pound or Play/Stop key.

NOTE: You can omit entering the bookmark number in which case Stratus will assign the first available bookmark number.

#### End Highlight Bookmark

After setting the starting position, navigate to the point where you wish to mark the end of the highlight.

Press the Bookmark once. You will hear “End highlight bookmark”. Press the Pound key or Play/Stop key to confirm. The current position is set as the ending position. If the end position is placed before the start position, they will be switched. The message "Highlight bookmark "X" inserted" is then played. The user can also cancel the highlight bookmark by pressing the Star (Cancel) key. In both cases, the state reverts to Start.

NOTE: When a highlight bookmark starting position is set but not the ending position, its highlight bookmark number becomes unavailable. Stratus will announce “Bookmark "X" already exists”  if the user enters a regular bookmark with this number.

NOTE: If you remove the book or power off the Stratus before closing the highlight, highlight will be auto-completed. If this occurs, the current location will be used as the end highlight position.

#### Go to Highlight Bookmark

When going to a highlight bookmark, if you end the bookmark number entry with the Play/Stop key, playback will start from this current position and stop at the highlight bookmark ending position. If the Pound (Confirm) key was used, Stratus will position at the bookmark but not play. While playing the highlighted passage you can jump to the beginning by tapping the Rewind key. At the end of the highlight you can play it again by tapping Rewind.

#### Highlight Bookmark List

When a highlight bookmark is reached with the bookmark list, playback will stop at the highlight bookmark ending position. A highlight that was started but hasn’t been closed yet will not be listed.

#### Remove Highlight Bookmark

When deleting a highlight bookmark, the "Highlight bookmark "X" removed" message will be announced. It is possible to delete a highlight bookmark that was started but not closed. This action reverts the highlight bookmark state to Start.

### Bookmark List

Press the Bookmark key four times or until you hear ‘bookmark list’.

Press the key 4 or key 6 to go from one bookmark to the next. Each time you press the key, you will hear the number of the next bookmark and playback will begin. Press the Pound key to confirm the current position and continue play.

Press the Star key to exit the Bookmark list.

NOTE: The bookmark list always contains one bookmark for the beginning and end of book. Bookmarks are sorted by their position in the book not by the bookmark number.

### Remove Bookmark

Press the Bookmark key five times or until you hear ‘remove bookmark’. Enter the number of the Bookmark you wish to remove. Press the Pound key to confirm.

To remove all the Bookmarks in a book, first press the Bookmark key five times or until you hear ‘remove bookmark’, then press the key 9 five times (enter number 99999). Then press the Pound key to confirm.

### Bookmark alert

While playing a book, you can have the Stratus alert you when you pass over a previously set bookmark. Bookmark alerts are disabled by default.

Bookmark alerts can be enabled or disabled. Read the Configuration menu section for more information.

## Text Search

The Stratus has a text search function for files in the Text Bookshelf. You may also search DAISY or NISO books found in the Talking Books bookshelf providing the book has text and the player is in text-to-speech mode (key 9). DAISY or NISO books that are recorded only cannot be searched as they have no text. Using the multi-tap text entry method on the number keys as you would to enter text messages on a mobile phone, you can type in your search item. If you have a multilingual Stratus, the typing keypad will offer the characters of the currently selected text-to-speech language. Press and hold key 7 to switch from one text-to-speech language to the other.

Follow these steps to use the search function:

- Open a text file from the Text bookshelf. Or, go to the Talking Books bookshelf and open a text-only book (e.g. Bookshare) or that has both recorded audio and text. If it has both recorded audio and text, make sure that you are in text-to-speech playback mode by pressing key 9.

- Press the Go To key multiple times until you hear, “Search”. The number pad is now set for text entry.

- Press the number keys 0 to 9 to type the text. For example, key 2 is used for letters a, b, c, and 2. Key 3 has d, e, f, and 3. Key 6 has m, n, o, 6 and so on. The space character is on key 0. Punctuation and special characters are on keys 0 and 1. To enter a letter, press the number key associated to that letter the appropriate number of times. When entering successive letters on the same key you need to pause until Stratus announces the letter. For example, to type the word, “cab”, you press key 2 three times, then pause until you hear, “c”, then press it once, pause until you hear, “a”, then press it twice to type the last letter, “b”.  You do not need to pause to type successive letters that are on different keys. For example, to enter the word, “mama”, you can press 6262 as quickly as you like.

- You may press the Sleep key to enter the Key Describer mode. Then press other keys to announce their function including announcing the characters for each number key. Press the Sleep key again to exit the Key Describer and return to Text Entry mode.

- To announce what you have typed, press the Fast Forward key.

- To erase the last character typed, press the Rewind key.

- When you finish typing, you can exit the text entry mode in 3 ways: Press Star (Cancel) to exit without searching, or press Pound (Confirm key) or Play to start the search. Star (Cancel) will not affect any previous search information.

- If you pressed the Pound (Confirm) key, the Stratus will position itself at the location of the found search item and announce the line. If you pressed the Play key, it will move to the found search item and start playing. The search is case insensitive. The search will find partial words in which case it will position at the start of the word containing the search text. Accented characters are not matched to non-accented characters or vice versa. A search always starts from the current position in a book.

If searching a large file, you may hear “Please Wait” while the search is in progress. If you press the Star (Cancel) key while searching, the search function is stopped, the current position remains at the original starting position.

### Search Next or Previous

- Upon finding the search item, Stratus will add a Search item to the navigation levels of keys 2 and 8 and select that level automatically. So, upon finding the first occurrence of your text, you can simply press keys 4 or 6 to find the previous or next occurrence. The previous or next Search function will remain on the 2/8 key rotation until you close the book. You can press key 4 or 6 to search for the previous or next item while in play or stop mode.

- If a search forward is performed and the end of the book is reached, the message "End of book" will be announced and the search then continues from the beginning of the book down to the original starting position of the search.

- If a search backward is performed and the beginning of the book is reached, the message "Beginning of book" will be announced and the search then continues from the end of the book up to the original starting position of the search.

- If no match is found, Stratus will announce the search item was not found and return you to the original starting position.

- When you open a new book the previous search item is cleared.

### Search for Current Word

If you do not enter any search text, Stratus will search for the word you are currently positioned at in the text. For example, assume you are positioned at the word “HumanWare” in your text file. If you press the Go To key multiple times until you hear, “Search”, and then press Play or Pound (Confirm key) without typing a search item, Stratus will move to the next occurrence of “HumanWare” in your file. If you pressed the Pound (Confirm) key, it will stop there and announce the line. If you pressed the Play key, it will start playing. Note that you can press Fast Forward to spell the current word before pressing Pound (Confirm key) or Play. Also, pressing Rewind once or entering a letter will erase the whole word so you can enter your search item.

# Bookshelf Structure and Characteristics

For files on all bookshelves, the maximum file name length, including the file path, is 512 characters (includes media/book path). When scanning the memory card at insertion, any folder or file with a full path name greater than 512 will be ignored.

The following sections provide general rules and information for listening to the bookshelves.

## Talking Books

### Talking Books Structure

Talking Books are DAISY, NISO NIMAS or EPUB type books and are saved in the $VRDTB bookshelf folder. DAISY and NISO require one folder per book. EPUB books are single files. The following list provides an example of how book files may be grouped in folders. There can be up to 8 levels of folders.

Folder: \$VRDTB.

Rules: Each EPUB file or DAISY folder is defined as a book with its own reading position and bookmarks. Books can be deleted using key 3 while browsing the bookshelf.

File: \$VRDTB\Book1.EPUB

Rules: This is an EPUB book in the root of $VRDTB.

Folder: \$VRDTB\Book1

Rules: This is a folder containing a set of DAISY or NISO book files. Thus, it defines a DAISY or NISO book in the root of $VRDTB.

Folder: \$VRDTB\Drama.

Rules: This is an optional folder to categorize the type of book. It will contain further subfolder categories, EPUB books, or DAISY/NISO book folders.

Folder: \$VRDTB\Drama\BookA..EPUB

Rules: This is an EPUB book file in the Drama category.

Folder: \$VRDTB\Drama\BookB.

Rules: The folder containing the DAISY or NISO book files. The entire folder is defined as the book

Folder: \$VRDTB\Mystery.

Rules: This is an optional folder for categorizing type of book.

Folder: \$VRDTB\Mystery\BookC.

Rules: Same as Book B.

### Talking Books Characteristics

Bookshelf Browse: Move between folder levels with keys 2 and 8. Note that the lowest folder level is also the book level since a book is defined as a collection of files in the lowest level folder. Higher level folders are optional for categorizing types of books as in Drama folder above. There can be up to 8 levels of folders. Use Go To Book to jump to a specific book number. Use key 3 to delete a book. A book can only be deleted if you are at the book level.

Reading: Navigate the book using keys 4 or 6. Choose navigation level with keys 2 and 8. Headings, page, phrase, and Time Jump navigation levels are available. Go To Page and Go To Heading are available.

Info (key 0): Reports information for the current book.

Where Am I (key 5): Reports the percentage within the whole book, the current heading number and name, current page, as well as the time (total, elapsed and remaining) for the book.

## Other Books

### Other Books Structure

The following list provides an example of how book files may be grouped in folders. Folders are numbered in the order in which they are found starting with the root, assuming it contains files.  There can be up to 8 levels of folders.

Folder: \$VROtherBooks.

Rules: Each file in the root is defined as a book with its own reading position and bookmarks. The single book files in the root can be deleted using key 3 while browsing the bookshelf.

Folder: \$VROtherBooks\Drama.

Rules: This is an optional folder to categorize the type of book.

Folder: \$VROtherBooks\Drama\BookA.

Rules: This is the folder containing the book files. The entire folder is defined as the book with one reading position and bookmark set. While browsing the bookshelf you may use key 3 to delete the book which means the entire book folder will be deleted.

Folder: \$VROtherBooks\Drama\BookB.

Rules: Same as Book A.

Folder: \$VROtherBooks\Mystery.

Rules: This is an optional folder for categorizing type of book.

Folder: \$VROtherBooks\Mystery\BookC.

Rules: Same as Book A.

### Other Books Characteristics

Bookshelf Browse: Move between folder levels with keys 2 and 8. Note that the lowest folder level is also the book level since a book is defined as a collection of files in the lowest level folder. Higher level folders are optional for categorizing types of books as in Drama folder above. There can be up to 8 levels of folders. Use Go To Book to jump to a specific book number. Use key 3 to delete a book. A book can only be deleted if you are at the book level.

Reading: Playback loops between the files within a folder (book). Navigate between files in a folder using keys 4 or 6. File and Time Jump navigation levels are available. Go To File and Go To Time are available.

Info (key 0): Reports information for the current book (folder).

Where Am I (key 5): Reports the percentage within the whole book, the current file number and name as well as the time (total, elapsed and remaining) for the current file.

## Music Bookshelf

The music bookshelf is in the reserved folder $VRMusic. It contains a folder structure of audio files and this entire structure is defined as the All Music book. Unless you create playlists, this will be the only Music book. If you add playlist (M3U) files to the root of $VRMusic then each playlist will be an additional music book.

### Music Structure

You can navigate up to 8 levels of folders in music. Use the 2 and 8 keys to select the folder or file navigation level. Then you can move backward or forward at the chosen level with the 4 and 6 keys. If you have more than 8 folder levels, those additional levels will be added to the level 8 list of folders.

If you select the ‘File’ navigation level, you will hear the name of the current file. If you select any folder Level navigation, you will hear the name of the current folder at that level. At any point in your structure you can always select the deepest navigation level. If the current folder is at a higher level than your deepest level then the folder name is not announced. For example, let’s say you have a music structure that contains $VRMusic\Classic\Mozart and $VRMusic\Classic\Beethoven\Ninth_symphony. If you are currently located in Classic\Mozart while playback is inactive, changing the navigation from File to Level 1 will announce ‘Classic’. Changing from Level 1 to Level 2 will announce ‘Mozart’. Changing from Level 2 to Level 3 will announce nothing since there is no folder name for a level 3 folder in Classic\Mozart. If located in Classic\Beethoven while playback is inactive, changing from Level 2 to Level 3 will announce ‘Ninth_symphony’.

If you place any files in the root of $VRMusic then they will be listed only at the file navigation level.

### Music Characteristics

Bookshelf Browse: Use keys 4 and 6 to move between each Music book or Go To Book to jump to a specified book number. Note that only 1 music book (All Music) will be on the music bookshelf unless you create additional playlist books. You cannot use key 3 to Delete a Book.

Reading: If loop is enabled, playback loops within the files of a music book. Playback will continue from the last file of one folder to the first file of the next folder. You can use keys 2 or 8 to select folder or file navigation. Go To Folder and Go To File functions are also available. If the player is set for Random play then the files in the current music book will be shuffled. For example, if the All Music book is being played all of your music files will be shuffled. If a Playlist book is being played then only the files in that playlist will be shuffled. When the end of a shuffled music book is reached, playback will start at the beginning of the shuffled music book.

INFO (key 0): Reports information for the current music book such as total folders and files and total times for the book.

Where Am I (key 5): Reports current position and information for the current file. Pressing Key 5 twice within 10 seconds will announce tag information data. If you don't want to hear all the tags, press Play to interrupt and resume playback.

Speed: Stratus will reset the variable speed control to normal speed and return to the previous speed setting when you exit the Music Bookshelf. Note: It is only possible to listen to stereo music at normal speed. If you speed up or slow down the music it will play in mono.

### Playlists

The Music Bookshelf may contain playlists. A playlist is a text file containing a list of supported audio files to play. It must have the "m3u” file extension. A playlist cannot link to other playlists.

### Temporary Playlist

You can create a temporary playlist for a subset of folders found in the $VRMusic folder. You cannot create a temporary playlist from the files found at the root level of the $VRMusic folder.

- Press key 1 multiple times until you reach the Music bookshelf. Then press the Go To key multiple times until you hear “Create Temporary Playlist”.

- Now use keys 2/8 and 4/6 to navigate to the first folder you want in your playlist. Then press Pound (Confirm key) or Play keys. The folder you selected and all the subfolders below it will now form your temporary playlist. If you pressed the Play key, the playlist will start to play.

Note: Selecting a folder for the creation of a temporary playlist works slightly differently than the folder navigation described in the above sections. Immediately after hearing the message “Create temporary playlist” you will hear the name of the first folder at the top level in your music book. Use keys 4 and 6 to cycle through all the folders at that top level. Once you have reached the top level folder of interest, you can use key 8 to go down a level and then, with keys 4 and 6, you will cycle through the folders at the second level inside that top folder. You can go to lower levels in the same fashion. In order to go to a different folder at a higher level, you will have to first use key 2 to go up to that level before using again keys 4 and 6 to navigate to it.

- Navigate within the playlist using keys 2/8 and 4/6, Go To Folder, and Go To File keys.

- Press key 0 to find the total number of folders and files and length of your playlist.

- Press key 9 to randomize the playlist and press key 9 again to turn off random play.

- The playlist is cleared as soon as you press key 1 to return to the bookshelf.

For example, let’s say you have organized your $VRMusic files into 3 levels of folders. You could have the music genre at level 1, artists at level 2 and albums at level 3. You can now easily play only music of a particular genre, or all the albums of a particular artist, or listen to a single album.  Suppose your level 1 genre folders included Classical, Country, Folk, and Rock. Now, suppose, within Classical, you have level 2 folders for the artists, Beethoven, Mozart, and Strauss. You decide you want to play all your Mozart music. Go to the Music bookshelf. Press Go To Key until you hear, “Create Temporary Playlist”, press key 6 until you hear, “Classical”. Now press key 8 to move down to your level 2 folders and press key 6 to reach the Mozart folder. Press the Play key. All your Mozart albums will now be used to create your temporary playlist and it will begin playing. If you press key 9, the tracks found in those albums will play in random order. Pressing Key 0 will announce the total number of Mozart albums, total files, and their total play time.

### Folder and File Name Announcement

When browsing the Music bookshelf, you may or may not want to hear file and folder name announcements. The Stratus will provide this information if the player is in Stop mode but will omit the announcements if the player is in Play mode. In either case, it will repeat the announcements if you press the key 5 Where Am I.  Thus, if you wish to use key 4 or 6 to navigate back and forth between music files and do not want to hear the folder/file announcements simply press the Play key first to put the player in play mode.

## Podcasts Bookshelf

The Podcasts bookshelf contains a folder structure of audio files where each folder represents a podcast feed and each file an episode of that feed. Each file (episode) is defined as a separate podcast book.

### Podcast Structure

The following list provides an example of how podcast files may be grouped in folders. There can be up to 8 levels of folders.

Folder: $VRPodcasts

May contain files: Yes

Rules: Any files found in the $VRPodcasts root will be announced as individual books.

Folder: \$VRPodcasts\FeedA.

May contain audio files: Yes.

Rules: This is the folder for grouping the podcast episode files under their feed name.

### Podcasts Characteristics

Bookshelf Browse: Move between folder levels of the Podcast structure using keys 2 and 8. The lowest level is the book (file) level. Files in the root of $VRPodcasts will only be announced at the book level. Use keys 4 and 6 to move between each Podcast book file (episode) or Go To Book to jump to a specified book number. Use key 3 to Delete a Book file. A book can only be deleted if you are at the book level.

Reading: Playback stops at the end of each file. Only Time Jump navigation level is available on keys 2 or 8. Go To Time is also available.

INFO (key 0): Reports information for the current Podcast book (file).

Where Am I (key 5): also Reports current position and information for the current file. Pressing Key 5 twice will announce tag information data. If you don't want to hear all the tags, press Play to interrupt and resume playback.

## Text Bookshelf

The Stratus can also store text files grouped in folders under the $VRText reserved folder. They are played with the Stratus text-to-speech. Each text file within the $VRText folder is considered a book. Supported formats include: txt, html, xml, rtf brf, bra ses3 and DOCX. Note that when present, image description tags will be announced in html and xml documents.

### Text File Structure

The text files can be either in the root of $VRText or organized into sub-folder categories. There can be up to 8 levels of folders.

### Text File Characteristics

Bookshelf Browse: Move between folder levels of the Text Files structure using keys 2 and 8. The lowest level is the book (file) level. Files in the root of $VRText will only be announced at the book level. Use keys 4 and 6 to move between each text file or Go To Book to jump to a specified file number. Use key 3 to Delete a text file. A book can only be deleted if you are at the book level.

Reading: Playback stops at the end of each file. The navigation levels on keys 2 or 8 include: page (if page markers are in the file), screen (if no page markers), paragraph, line, sentence, word, spell, and character. If you do a search in text then a Search level is added as the last navigation level to allow you to find the next or previous search item. Go To Page is available if the file has page markers. Go To Percent (see below) is also available.

INFO (key 0): Reports information for the current text file such as its size.

Where Am I (key 5): Reports current position as a percentage from the start of the file.

### Navigating HTML/XML/DOCX Headings

For html/xml files that have heading tags (<H1> to <H6>), these will correspond to equivalent navigation levels 1 to 6 on the 2/8 menu. Be aware that some html/xml files do not use these heading tags consistently. For example, the file may have an H1 and H3 heading with no H2 heading. In this case the Stratus will present only level 1 and level 3 on the 2/8 menu. Also, an html/xml file could have a single H1 heading for a title with no other H1 tags in the file. In this case, Stratus would jump to the end of the file if you pressed right arrow (key 6) after choosing level 1 navigation.

# Configuration Menu – Key 7

The Stratus can be customized using the Configuration menu.

To open the Menu, press the 7 (Menu) key. Pressing the 7 (Menu) key again will switch to the next Menu. Browse Menu Items with the 2 and 8 (Up/Down)  keys, use the 4 and 6 keys (Left/Right) to select an Item’s value and press Pound key to confirm. Press Star (Cancel) or any other key to exit the Menu.

## Menus and Menu Items List

The available menus, menu items and values are listed in their respective order below. Note that the first listed values in the list are the factory default settings.

Menu: Navigation & Playback.

Item: Playback.

Values: Loop (disable or enable).

Item: Time jump.

Values: 1, 5, 10, or 30 minute Time Jump for navigation menu on 2/8 (Up and Down) keys.

Item: Key beeps

Values: enable, disable

Item: Sleep messages

Values: enable, disable

Item: Bookmark Alert

Values: enable, disable

Menu: Skip.

Item: All.

Values: Play, Skip, On Demand, Custom.

Item: Footnote.

Values: Play, Skip, On Demand.

Item: Page number.

Values: Play, Skip.

Item: Producer note.

Values: Play, Skip, On Demand.

Item: Sidebar.

Values: Play, Skip, On Demand.

Menu: Language.

Item: Text to speech language

Values: Toggle between available languages

Item: Braille Table

Values: Alternate between available Braille tables

Item: Code page (if applicable for the language)

Values: Alternate between available Code page tables

Menu: Format media

Item: Format SD card

Value: Confirm format.

### Playback

#### Loop

When selecting the Loop mode, Playback will automatically restart again at the beginning when reaching the end of a book.

#### Time Jump

This option allows you to set the Time jump length. When reading a book, you can select the Time jump navigation option using keys 2/8 (Up and Down). You will be able to navigate back and forth, using keys 4/6 (Left and Right), in the text by the Time jump value selected.

#### Key beeps

You can enable or disable the beeps heard when pressing keys. Disabling this option will also suppress the second beep heard when starting playback. The key beeps are enabled by default. The Tone, Volume, and Speed keys will only beep at their minimum, normal, and maximum positions regardless of this menu setting.

#### Sleep messages

The Sleep timer's 1 minute warning and Sleep off messages can be enabled or disabled. They are enabled by default.

#### Bookmark Alert

If this option is enabled, a message will be heard when a bookmark is encountered. Each message is followed by the bookmark number.

### Skip

The Skip Menu contains the configuration options for the skippable elements. Play is the default value. Here is a list of menu items and their respective values:

All.

Values: Play, Skip, On Demand, Custom.

Note: You cannot select Custom. It is set automatically when you modify any of the skippable elements.

Footnote.

Values: Play, Skip, On Demand.

Page Number.

Values: Play, Skip.

Production Note.

Values: Play, Skip, On Demand.

Sidebar.

Values: Play, Skip, On Demand.

# Battery Replacement Procedure

Contact HumanWare or your authorized distributor to obtain replacement batteries.

## Removing spent battery

Place the Victor Reader Stratus in front of you on a table or flat surface.

Flip the Victor Reader Stratus upside-down so as to have the underside of the device facing you. Position the player so as to have the carry handle away from you.

Locate the battery compartment cover. It is near the carry handle opening. There are 2 sliding tabs. These tabs have 3 tactile ridges. The battery cover is located just below these two sliding tabs. To open the battery cover, simultaneously push the two sliding tabs away from each other using your thumbs. The battery cover should pop up, allowing you to remove the battery.

## Inserting new battery

Victor Reader Stratus batteries are designed to be mistake-proof, meaning that they can only be inserted one way.

To properly insert it into the compartment, first place the battery flat in front of you.

The battery is rectangular. One of the 2 large surfaces has a label, this is the bottom surface.  2 of the 4 lateral sides are completely flat. The other 2 lateral sides have tabs and connectors.

One has a single tab, in the middle, extending from the top surface of the battery. The opposite side has a connector in middle and two tabs on either side of it.

To install battery, insert side with the two tabs and connector. The tabs and connector should fit into their corresponding openings. Then press down to lock the battery into place.

Note that a new battery is usually at 50% of its full capacity and need to be recharged completely by connecting Stratus to a Power outlet.

# Updating Stratus Software

HumanWare may from time to time offer new versions of the Stratus software.

The software update is a .UPG file downloaded from HumanWare's website.

To update your Stratus:

- Burn the .UPG file onto a CD or copy the file to an SD card or USB stick.

- Connect the Stratus to a power outlet.

- Power on the player and insert the CD or SD card/USB stick.

- The update will start and report the new version number that is being installed. An installation progress message will be heard every 10 seconds.

- After the update is done, the player will power off.

- If update was done using a CD, it will be automatically ejected the next time the Stratus is powered on.

# Troubleshooting

## Book Navigation

- Why does Victor Reader Stratus not always announce the same DAISY levels when I press the Up and Down keys?

The Up and Down keys cycle through the DAISY navigation levels provided by the producer of the book. Not all DAISY books have the same navigation levels.  For example, your book’s producer may have only inserted level 1 (often used for chapters) marks into the book.  In this case, the Up and Down keys will announce only "level 1" and "phrase".  A phrase level is always included.  Victor Reader Stratus can only navigate at the levels provided by the book producer. Also, the phrase jump is not the same for each book.  Some producers may include a phrase mark for a single sentence while others may markup phrase as an arbitrary time jump.  Still others have converted older cassette books and inserted a phrase as an entire side of one cassette.  For books with little or no DAISY navigation marks, you will need to navigate with the Fast forward and Rewind keys.  The Rewind key is to the left of the Play/Stop key and the Fast Forward key is to the right of the Play/Stop Key.

- How do I know what DAISY navigation levels are available in my book?

The Victor Reader Stratus Up and Down keys will cycle through the levels of navigation available in the book.  Many book producers include a section at the beginning of the book that describes what the DAISY levels correspond to in that book.  Check with your book producer if you are unsure of the book’s navigation features.

- How do I quickly jump to the start or end of a book?

Press the Go To key.  When prompted for a number, press 0 or Rewind to jump to the beginning of the book, or press Fast Forward to jump to the end of the book.

## Battery and Charging

- Can I damage my battery if I leave the power cord connected for a long time?

No.  However, if you are not going to use your Victor Reader Stratus for a long time such as being away on vacation it is best to disconnect it from the power mains to avoid potential damage from power surges.

- Can I replace my battery?

Yes. Replacement batteries can be obtained from HumanWare or authorized dealers.

- How many times can I recharge my battery before it requires replacement?

The battery is rated for 300 charge/discharge cycles.  After that it will still recharge but its capacity to retain a charge will decrease.  When the battery no longer maintains a charge it should be replaced.

- What precautions should be taken if I am not planning to use the player for an extensive period of time?

For a period of time exceeding 3 months, it is best to remove the battery from the player. The player and batteries should be kept within a temperature of -20 to 35 Celsius and humidity levels between 5 and 90% without condensation.

## CDs

- What should I do if my CD is stuck and will not eject?

First, try turning Off your Victor Reader Stratus and then turning it back On and then pressing eject.  If the On/Off key appears to be not working (no beep is heard when you press it) then hold the On/Off key down for 10 seconds, then release it.  Then hold down the Eject key while you again press the On/Off key.  Continue to hold the Eject key until you hear, "Welcome To Victor Reader". Victor Reader Stratus should then eject the CD.

- Can I damage my Victor Reader Stratus if I insert the CD upside down?

No.  Victor Reader will simply eject the CD.

- Why does my Victor Reader Stratus make whining or chirpy sounds every couple of minutes?

This is the normal sound of the CDROM drive while the player is reading information from the CD.  For DAISY CD books, after the CD is read it will spin down to save power and become silent for several minutes until it needs to be read again.  Pressing navigation keys or jumping to a bookmark will also cause the CD to spin-up in order to be repositioned to the new location.

- What should I do if my CD skips or stops playing?

Poor quality CDs or CDs with fingerprints or scratches will often not play properly or not play at all. Eject the CD. Try cleaning it using a soft lint free cloth or CD cleaning cloth, which can be purchased at most stores that sell CDs.  Do not use paper towel or tissues. Clean the surface of the CD from the middle towards the outer edge in straight lines. Do not use a circular motion.  If the CD still does not play, it should be inspected for scratches. Scratched CDs should be returned for replacement.

- Do I need to clean the CD drive or laser reading lens?

No.  The type of drive used in the Victor players does not require cleaning.

- May I attach a Braille or other label to the CD?

No.  The label can adversely affect the balance of the CD as it spins at high speed.  Also, if the label should come loose inside the player it will very likely damage the CD-ROM drive. If you receive a CD with a Braille or other non-manufacturer’s label you should remove it.

- Why do I sometimes hear the message, "Please Wait", when I press a key even though there is a CD loaded?

This may happen if you press keys too soon after inserting the CD or turning On the player.  Victor Reader Stratus requires some time to scan the disk and load the DAISY CD navigation information.  After turning your player On or inserting a CD, you should wait until Victor Reader Stratus announces the book title before you press any keys.

- What does the message, "No book on media" mean?

This is the message Victor Reader Stratus issues if it does not recognize the type of CD.  Stratus can only play DAISY, MP3, or audio CDs.

- What does the message, "Audio format not supported" mean?

The CD has audio files that Victor Reader Stratus recognizes but the files are formatted in a way that is incompatible with the Victor Reader Stratus software.  You should notify HumanWare technical support.

- Can I damage my Victor Reader Stratus by playing a scratched CD?

No. The CD reading lens does not come in contact with the CD surface so it will not be damaged if the CD is scratched. It will also not be subject to wear from repeated plays as with cassette players.

## General

- What should I do if the On/Off key is not working?

If no beep is heard when you press the On/Off key first ensure the player is plugged into the AC power source.  If the On/Off key is still not working, press and hold it down for 10 seconds.  This will reset the player.  Then press it again and Victor Reader Stratus should beep and start normally.

- Can the internal Victor Reader Stratus software be updated?

Yes.  When HumanWare upgrades the Victor Reader Stratus software, customers are able to install the new software by simply inserting a special upgrade CD into the player, which automatically refreshes its internal software program. The upgrade CD may be obtained from HumanWare customer support, directly downloaded from the HumanWare website, or from some dealers who provide this support.

- Should I delete bookmarks to save memory space?

No. Victor Reader Stratus can store over 1500 bookmarks spread over many books. Even if you exceed this large number Stratus will replace the bookmarks that were used the least often. So, you only need to delete bookmarks if you do not want them.

# Technical Specifications

- Dimensions without cover:	22. X 21.6 X 4.8. cm (8.7 X 8.5 X 1.9 in.)

- Dimensions with cover	22.8 X 22.2 X 5.1 cm (9 X 8.7 X 2 in.)

- Weight:			0.95 kg  (2.1lbs) with battery

- DAISY Formats Supported:	DAISY 2.0 and 2.0.2

DAISY NISO Z39.86 2002 and Z39.86 2005

- Battery type:		Rechargeable Li-Polymer battery. 8.4 volts 1650 mAh

- Charging time:		4 hours

- Operating time:		10 hours for continuous play of DAISY MP3 book

- Power Supply:		The supplied power adapter meets power requirements only for

the country in which it was sold.  Check with your dealer for

alternate power adapters used for other countries.

- Coding Algorithms:		AAC, AMR-WB+, FLAC, MP3, Ogg Vorbis, Speex, WAV

- Text Files: BRA, BRF, DOCX, HTML, RTF, SES3, TXT, XML

- Text-To-Speech: Acapela for Linux Embedded

- Power jack			2.5 mm (interior)/ 5.5 mm (exterior)

- earphone jacks		3.5 mm

- Equipment Classification (IEC 601-1)

- Type of Protection: Class 1

- Applied Parts: None

- Sterilization Method: Not intended for sterilization

- Enclosure Degree of Protection: IP20

- The equipment is not AP or APG category

- Mode of Operation: Continuous

- Environmental

- Operating Temperature 10 to 40º C

- Transport & Storage Temperature -20 to 60º C

- Operating Humidity 20% to 80% (RH)

- Transport & Storage Humidity 10% to 95% (RH)

- Operating Altitude -200 to 2000 m

- Transport & Storage Altitude -200 to 12 000 m

- Transport & Storage Barometric Pressure 70-103 kPa

For additional information, please contact HumanWare:

Service

1800 Michaud

Drummondville, Québec

Canada J2C 7G7

Telephone: 1 (819) 471-4818

Toll-free (Canada & USA): 1 (888) 723-7273

Fax: 1 (819) 471-4828

Website: www.humanware.com

# Customer support

For customer support, please contact the HumanWare office nearest you or view our web site at: www.humanware.com

North America: 1(800) 722-3393 or send an email to us.support@humanware.com

Europe: (0044) 1933 415800 or send an email to eu.support@humanware.com

Australia / Asia: (02) 9686 2600 or send an email to au.sales@humanware.com

# Manufacturer Warranty

This device is a high quality product, built and packaged with care. All units and components are guaranteed against any operational defects as follows:

US and Canada: One (1) year

Continental Europe and UK: Two (2) years

Australia and New Zealand: One (1) year

Other countries: One (1) year

Warranty covers all parts (except battery) and labor. If any defect should occur, please contact your local distributor or the manufacturer technical assistance line.

Note: Warranty terms may periodically change, please consult our website for the latest information.

Conditions and Limitations:

No replacement or repair covered by the warranty will be carried out unless the unit is accompanied by a copy of the original bill of purchase. Please retain your original. If the unit has to be returned, please use the original packaging. This warranty applies to all cases where the damage is not a result of improper use, mistreatment, negligence or acts of God.

# End User License Agreement

By using this product (Victor Reader Stratus) you agree to the following minimum terms.

      - License Grant. HumanWare grants to End User a non-exclusive, non-transferable right and license to use the Software on this product.

      - Ownership of Software. End User acknowledges that HumanWare retain all right, title and interest in and to the original, and any copies, of software which is incorporated into this product. End User agrees not to: modify, port, translate, decompile, disassemble, reverse engineer, or make public in any way the software of this Product.

This product includes software developed by the OpenSSL Project for use in the OpenSSL Toolkit (http://www.openssl.org/)

# Appendix 1 - Status Error Messages

SYSTEM error 1: CDROM, Error while accessing the CDROM DRIVE.

SYSTEM error 2:  MEMORY, Memory allocation problem.

SYSTEM error 3: BATTERY, Error with battery detection.

SYSTEM error 4: INT_STOR_FAIL, Error detecting internal storage

SYSTEM error 5: INT_STOR_CORRUPT, Corrupted internal storage

MEDIA error 1: READ, Error reading sector on disc.

MEDIA error 2: FORMAT, Unsupported File system.

MEDIA error 3: MOUNT, Error while mounting device.

MEDIA error 4: AUDIO, Does not apply.

MEDIA error 5: FILE, Error accessing the audio file.

MEDIA error 6: WAV, Error creating new wav structure.

MEDIA error 7: MPEG, Error handling an mpeg file.

MEDIA error 8: VORBIS, Error handling a vorbis file.

MEDIA error 9: NO AUDIO, No audio output available.

MEDIA error 10: AMR-WB+, Error handling a AMR-WB+ file.

MEDIA error 11: MP4-AAC, Error handling a MP4-AAC file.

MEDIA error 12: WRITE, Unable to open file for writing.

MEDIA error 13: SDLOST, card removed during write operation.

MEDIA error 14: USBLOST, USB device removed during write operation.

BOOK error 1: GENERIC, Unspecified.

BOOK error 2: MP3, Error with the mp3 decoder.

BOOK error 3: VORBIS, Error with the ogg vorbis decoder.

BOOK error 4: WAV, Error with the wav decoder.

BOOK error 5: NO FILE NAME, No file name available.

BOOK error 6: NO CUR POS, No current position is set.

BOOK error 7: FILE NOT FOUND, File not found on MCART.

BOOK error 8: AMR-WB+, Error with the AMR-WB+ decoder.

BOOK error 9: Audible.com DRM, Player unauthorized to play a book due to DRM.

BOOK error 10: VRTEXT, Error parsing text file.

BOOK error 11: TEXT SIZE, Text file is too big.

BOOK error 12: Braille translation error.

BOOK error 13: MP4-AAC, Error with the MP4-AAC decoder.

BOOK error 14: Re-parse text, Error re-parsing text file.

BOOK error 15: STRUCT_SIZE, Text book structure too big.

Note: some messages may not apply to all player models.